LQ35L 3.0
Thermal Monocular

With a basic magnification of 1.9x, a 640x512 detector, and a 35mm F1.0 lens, a 1000m LRF，the LQ35L 3.0 offers unparalleled performance for professional hunters and mid-to-long-range applications. Its 7.5° (13.1m at 100m) field of view and a detection range of up to 1800m ensure exceptional detail in vast, open spaces, making it the ultimate choice for users seeking premium quality with proven shutterless technology.

- Key Feature

- Up to 640×512 Thermal resolution

- High-sensitivity thermal module with NETD ＜ 15 mK (@25°C, F# = 1.0).

- 35 mm, F1.0 lens with thermal detection range up to 1800 m

- HIKMICRO Shutterless Imaging System (HSIS)

- AI-powered Image Pro 3.0 and Zoom Pro algorithms

- 0.49-inch OLED display with 1920 × 1080 resolution

- Up to 1000m Laser Range finder

- Replaceable and rechargeable 18650 battery provides up to 4.5 hours of continuous operation at -20°C

| - Specification |
| --- |
| Thermal Module |  |
| Image Sensor | VOx Uncooled Focal Plane Arrays |
| Max. Resolution | 640x512 |
| Frame Rate | 50Hz |
| Pixel Interval | 12μm |
| Response Waveband | 8μm to 14μm |
| NETD | Less than 15 mK (@25°C),F#=1.0 |
| Lens (Focal Length) | 35mm, f1.0 |
| Focus Mode | Focus Ring |
| Detection Range | 1800m |
| Min. Focusing Distance | 3.5m |
| Magnification | 1.9x - 15.2x (8x) |
| Field of View(H × V), Degrees / m @100m | 12.5°×7.5°/21.9m×13.2m |
| Image Display |  |
| Display | 1920x1080, 0.49 inch, AMOLED |
| Palettes | Black Hot, White Hot, Red Hot, Fusion, Red Monochrome, Green Monochrome |
| Exit Pupil | 6mm |
| Eye Relief | 20mm |
| Diopter (Range) | -5D to +3D |
| Brightness Adjustment | Yes |
| Tone Adjustment | Cold, Warm |
| Contrast Adjustment | Yes |
| Zoom Pro | Yes |
| HSIS (HIKMICRO Shutterless Image System) | Yes |
| Image Pro 3.0 | Yes |
| System |  |
| Record Video | Yes |
| Capture Snapshot | Yes |
| Audio Recording | Yes |
| Standby Mode | Yes |
| Built-in Memory | Built-in EMMC (64 GB) |
| PIP | Yes |
| Hotspot | 2.4 GHz/5 GHz Wi-Fi Waveband |
| Hot Track | Yes |
| Distance Measurement | Yes |
| Local Album | Yes |
| Power Supply |  |
| Battery Type | Replaceable and Rechargeable Li-ion 18650 Battery |
| Battery Operating Time | 5-hour continuous running with hotspot on (@25°C) 4.5-hour continuous running with hotspot on (@-20°C) |
| Type-C Power Supply | 5V DC, 2A Supports External Power Supply, Support Direct Charging |
| General |  |
| Protection Level | IP67 |
| Dimension | 173.3 x 66.8 x 49.3 mm |
| Weight | 410.7g with battery |
| Working Temperature | From -30°C to 45°C (-22°F to 113°F); Humidity: 90% or Less |
| Laser Rangefinder |  |
| Safety Class for Laser | Class 1 |
| Wavelength | 905nm |
| Max. Measuring Range | 1200m |
| Measurement Accuracy | ±1m |
| Min. Measuring Range | 5m |

- Available Model HM-TSF6-35Y1G/W1L-LQ35L 3.0

- Dimension

- Accessory

- Included

HM-3632DC Battery

- Optional

| HM-OUTDOOR-POUCH Monocular Pouch | HM-RMB-LF HM-RMB-LF |
| --- | --- |